INT8 YOLOv8 NPU

System Architecture Design Document

Xilinx Kria KR260 (XCK26 ZU5EV)

Target: YOLOv8n INT8 Inference at 640×640

Design Status: In Progress — v2.1

# 1. System Overview

## 1.1 Platform

The target platform is the Xilinx Kria KR260 Robotics Starter Kit, built around the XCK26 SoC (Zynq UltraScale+ ZU5EV). The device provides a quad-core ARM Cortex-A53 Processing System (PS) and a programmable logic (PL) fabric on the same die, connected via high-speed AXI buses. Both share a single 4 GB DDR4 DRAM controller.

| Resource | XCK26 Specification |
| --- | --- |
| DSP48E2 slices | 1,248 |
| BRAM36 blocks | 144 |
| URAM288 blocks | 64 |
| LUT6 (approx) | ~117,000 |
| DDR4 capacity | 4 GB |
| DDR4 peak BW | 19.2 GB/s (PS AXI, both HP ports combined) |
| PS cores | Quad-core ARM Cortex-A53 @ up to 1.5 GHz |
| OS | Ubuntu Linux (on PS) |
| Remote debug | Tailscale + Vivado Lab Edition hw_server @ 3121 |

## 1.2 Design Goals

- Run YOLOv8n at 640×640 input resolution, targeting 20–30 FPS end-to-end

- Use INT8 quantization throughout with per-channel scale factors (one M/S pair per output channel)

- Run approximately 95% of FLOPs on the FPGA NPU; ARM handles only the Detect head, upsample, and concat

- Hand-written Verilog RTL — no HLS, no Vitis AI overlay

- Hardware 2D strided DMA (no software im2col at any stage)

- Batch normalization folded offline into Conv weights and bias via model.fuse() — no BN hardware

## 1.3 Key Design Decisions

| Decision | Rationale |
| --- | --- |
| INT8 per-channel quantization | Per-channel (M, S) per output channel gives significantly better accuracy than per-tensor. BRAM coefficient buffer makes per-channel load essentially free. DSP48E2 dual-packing gives 2 INT8 MACs per DSP (WP486). |
| 300 MHz PL clock | 50% throughput uplift over 200 MHz with no additional area cost. Target ~102 GOPS from 256 MACs/cycle. XCK26 achieves 300 MHz on properly pipelined DSP48E2 chains. |
| Weight-stationary systolic array | Weights (3.2 MB) reused across spatial positions. Keeping weights stationary in PE registers minimises SRAM traffic per inference. |
| 2D strided DMA (no im2col) | Eliminates 40 MB/frame of DDR4 writes and all ARM im2col CPU time. Hardware 4D-stride address generator fetches patches in im2col layout directly to SRAM. Padding inserted in hardware. |
| Dedicated SRAM banks | Separate Act A/B, Weight A/B, Residual, and Output banks enable independent ping-pong for activations and weights, plus direct VPU residual access for C2f skip connections. |
| Fused BN + Requant | M and S coefficients encode the folded BN scale. Single Requant stage replaces separate BIAS_ADD + REQUANT pipeline, saving one instruction and one SRAM round-trip per layer. |
| Explicit PSB_ACC / PSB_FLUSH | Clean ISA boundary between MATMUL (compute) and PSB management (accumulation state). Sequencer does not inspect tile-counter state inside the MATMUL decode path. |
| HREDUCE in VPU | 4-stage horizontal tree reduction for DFL box decoding runs on-chip, reducing ARM detect-head compute and eliminating one DDR4 transfer per inference frame. |
| UPSAMPLE / CONCAT in DMA | FPN neck upsample and concat kept in hardware DMA — avoids ARM-NPU round-trips and leverages existing AXI HP burst engine. Saves ~2 ms/frame vs ARM NEON. |
| FENCE (not SYNC) | Bitmask-based FENCE handles multi-unit synchronisation (e.g. wait for DMA + PSB simultaneously). More expressive than a two-FIFO SYNC primitive for a five-unit pipeline. |
| Microcoded sequencer | 13 instruction types, 5 functional units — full soft-core is overkill. Microcode gives runtime flexibility without HDL re-spin. |

## 1.4 Block Inventory

| Block | Implemented In | DSPs | Key Function |
| --- | --- | --- | --- |
| Microcoded sequencer | Custom RTL | 0 | Fetch, decode, dispatch, dependency tracking |
| Dependency FIFOs | Xilinx FIFO Generator IP | 0 | RAW/WAR token queues between unit pairs |
| Systolic array 16×16 | Custom RTL | 256 (v1) / 128 (v2 dual-pack) | INT8 GEMM, weight-stationary |
| Partial sum buffer (PSB) | Custom RTL + BRAM | 0 | INT32 accumulation across K tiles; explicit PSB_ACC / PSB_FLUSH control |
| Requant pipeline | Custom RTL + BRAM | 64 | Per-channel INT32→INT8 with fused BN scale; BRAM coefficient buffer |
| SIMD VPU 64-lane | Custom RTL | 0 | SiLU LUT, ELEW_ADD, ELEW_MUL, RELU, MAXPOOL, HREDUCE (DFL) |
| Act ping-pong SRAM | Xilinx BRAM Generator IP | 0 | Banks A+B for double-buffered activations |
| Weight ping-pong SRAM | Xilinx BRAM Generator IP | 0 | Banks A+B for prefetched weight tiles |
| Residual SRAM | Xilinx BRAM Generator IP | 0 | C2f skip-connection tensors; direct VPU read port |
| Output SRAM | Xilinx BRAM Generator IP | 0 | VPU write-back; feeds DMA_STORE |
| 2D Strided DMA | Custom RTL | ~4 | 4D-stride address gen, AXI4 burst, upsample, concat, zero-pad insertion |
| AXI-Lite CSR block | Custom RTL | 0 | Configuration registers, sequencer kick |
| Detect head | ARM PS (software) | N/A | HREDUCE DFL assist, sigmoid, NMS — runs in PyTorch on A53 |

## 1.5 DSP Budget Summary

| Block | DSPs | Notes |
| --- | --- | --- |
| Systolic array 16×16 (INT8, v1 naive) | 256 | 1 DSP48E2 per PE; INT8×8 fits in 27×18 multiplier |
| Systolic array 16×16 (INT8, v2 dual-pack) | 128 | v2: 2 INT8 MACs per DSP using pre-adder trick (WP486) |
| Requant pipeline 64-lane | 64 | INT32 × INT16 M multiply per channel lane |
| DMA address arithmetic | ~4 | Stride accumulation |
| Total (v1) | ~324 / 1,248 | ~26% of device; 924 DSPs remain for future use |

# 2. Hardware Blocks

## 2.1 Microcoded Sequencer + Dependency FIFOs

The sequencer is the control brain of the NPU. It has no datapath and uses minimal LUTs. The ARM driver writes a packed stream of 128-bit instructions into a DDR4 instruction buffer, then writes a single CSR register over AXI-Lite to kick execution. The sequencer fetches instructions, decodes the opcode and unit target, and dispatches the payload to one of five per-unit command FIFOs (DMA, SA, PSB, Requant, VPU).

### Dependency FIFOs

Between each producer/consumer pair there is a pair of small (depth-4) token FIFOs: RAW and WAR. Each instruction word contains four dependency flag bits: push_next, push_prev, pop_next, pop_prev. This guarantees correct data ordering without shared-state arbitration. Modelled on Apache TVM VTA.

## 2.2 Systolic Array 16×16 (INT8)

A 16×16 grid of Processing Elements implementing weight-stationary dataflow. Each PE computes one INT8 multiply-accumulate per clock cycle.

### PE Datapath

- Weight register: 8-bit signed, loaded once at layer start via WT_LOAD and held stationary

- Input register: 8-bit signed activation, passes horizontally across the row

- Accumulator: 32-bit signed INT32, accumulates in_a × weight each cycle

- One DSP48E2 per PE (256 total in v1)

### Weight Loading

Weights flow into the dedicated Weight Bank via WT_LOAD (ping-pong with Weight Banks A/B). The next layer’s weights are prefetched into the inactive Weight Bank while the SA computes on the current layer. Loading a full 16×16 weight tile takes 16 cycles.

### Activation Alignment (Skew FIFOs)

Activations arrive diagonally via skew FIFOs of depth equal to the column index (0, 1, …, 15). Shallow Xilinx FIFO Generator instances.

### K-Tile Accumulation via PSB_ACC / PSB_FLUSH

For C_in > 16, MATMUL is issued multiple times. After each MATMUL, PSB_ACC accumulates the INT32 result into the Partial Sum Buffer. When all K-tiles are exhausted, PSB_FLUSH forwards the final INT32 values to the Requant pipeline. There is no last_k flag in the MATMUL payload — K-tile counting is managed entirely by the sequencer’s PSB instruction sequence.

## 2.3 Partial Sum Accumulation Buffer (PSB)

A 16×16 INT32 register file between the systolic array output and the Requant pipeline. Eliminates the SRAM read-modify-write hazard across K-dimension tiles.

| Property | Value |
| --- | --- |
| Size | 16 output rows × 16 output cols × 4 bytes = 1 KB (expandable to 16×64 for wider tiles) |
| Implementation | Inferred register file or 1× BRAM18 |
| PSB_ACC behavior | Adds new SA output to running INT32 total each call; no SRAM access |
| PSB_FLUSH behavior | Forwards final INT32 values to Requant; zero-clears PSB for next layer |

## 2.4 Requant Pipeline (Fused BN + Requant)

Converts INT32 PSB output to INT8 in a single pipelined stage using per-channel (M, S) coefficients pre-computed offline during PTQ. The M and S values simultaneously encode the folded BatchNorm scale, replacing a separate BIAS_ADD instruction.

Operation per channel: INT8_out = clip(round((INT32_acc × M) >> S), −128, 127)

| Component | Detail |
| --- | --- |
| Coefficient BRAM | 512 × {M: INT16, S: UINT4} — holds one pair per output channel (worst-case 512 ch) |
| COEFF_LOAD instruction | ARM writes (M, S) pairs via DMA into coefficient BRAM before each layer |
| Multiply path | INT32 × INT16 → INT48, right-shift by S, round |
| Clip | Saturating clamp to [−128, 127]; one DSP48E2 per lane for the M multiply |
| Throughput | 1 INT8 result per lane per cycle; 64-lane pipeline = 64 channels/cycle |

## 2.5 SIMD VPU 64-Lane

The Vector Processing Unit has 64 independent parallel lanes. It receives INT8 data from the Requant pipeline and applies a configurable chain of post-processing operations, writing INT8 results to the Output SRAM bank.

| Stage | Operation | Detail |
| --- | --- | --- |
| Act LUT | SiLU (or bypass) | 256-entry × 8-bit BRAM table per lane. INT8 input is the array index; output is SiLU(x) in one cycle. LUT_BYPASS mux passes value unchanged for linear layers. LUT pre-loaded per layer via LUT_LOAD. |
| Eltwise + MaxPool | ELEW_ADD / ELEW_MUL / MAXPOOL | Saturating INT8 lane-wise add for C2f residual connections (reads Residual Bank directly). SPPF three-stage 5×5 max-pool via MAXPOOL instruction. |
| RELU | Passthrough clamp | max(0, x) per lane. No DSP/LUT. Included for future YOLOv8 variants. |
| HREDUCE | DFL box decode | 4-stage binary reduction tree (16→08→04→02→1). One HREDUCE per frame: max → subtract → exp LUT → sum → dot·[0…15] produces continuous box coords. Passthrough on conv layers. |

## 2.6 SRAM Hub — Dedicated Banks

The on-chip SRAM is partitioned into six named banks. Dedicated banks eliminate port-conflict arbitration and allow independent ping-pong of activations and weights simultaneously.

| Bank | Function | Access |
| --- | --- | --- |
| Act Bank A | Ping-pong activation buffer (first slot) | Write: DMA; Read: SA activation input |
| Act Bank B | Ping-pong activation buffer (second slot) | Write: DMA; Read: SA activation input |
| Weight Bank A | Ping-pong weight buffer (first slot) | Write: DMA (WT_LOAD); Read: SA weight load phase |
| Weight Bank B | Ping-pong weight buffer (second slot) | Write: DMA (WT_LOAD); Read: SA weight load phase |
| Residual Bank | C2f skip-connection tensor | Write: DMA; Read: VPU eltwise stage (bypasses SA, PSB, Requant) |
| Output Bank | VPU write-back after SiLU/eltwise | Write: VPU; Read: DMA (DMA_STORE to DDR4) |

### BRAM Allocation (~38 BRAM36 total)

| Allocation | BRAM36 | Notes |
| --- | --- | --- |
| Act Banks A+B | ~16 BRAM36 | Holds tiled activation patches post 2D-strided DMA fetch |
| Weight Banks A+B | ~14 BRAM36 | Holds one 16×16 weight tile per bank (double-buffered) |
| Residual Bank | ~4 BRAM36 | C2f skip tensors; sized for largest skip (80×80×64 = 409 KB — may require URAM) |
| Output Bank | ~2 BRAM36 | VPU output buffer |
| Requant coeff BRAM | ~1 BRAM18 | 512 × (INT16 M + UINT4 S) |
| SiLU LUT BRAMs | ~1 BRAM36 | 64 lanes × 256 entries × 1B = 16 KB |
| Total | ~38 / 144 | ~26% of device BRAM |

## 2.7 2D Strided DMA Engine

The DMA engine replaces all software im2col. It is an AXI4 master connected to AXI HP0 (reads) and AXI HP1 (writes). It computes source addresses from a 4D-stride descriptor, fetching spatial patches directly in im2col layout. Zero-padding is inserted in hardware — no out-of-bounds reads ever reach DDR4.

Descriptor registers: {base_addr, row_stride, tile_w, tile_h, ch_count, pad_top, pad_bottom, pad_left, pad_right}

| Mode | Opcode | Description |
| --- | --- | --- |
| 2D strided fetch | DMA_LOAD | Fetch spatial tile from DRAM to Act Bank. Computes addresses as: addr = base + h*row_stride + w*ch_count + c. Inserts zero words at padded edges automatically. |
| Weight prefetch | WT_LOAD | Prefetch weight tile from DRAM to inactive Weight Bank. Runs concurrently with SA compute on active Weight Bank. |
| Store back | DMA_STORE | Burst-write INT8 Output Bank contents to DDR4 after VPU completes. |
| Upsample 2× | UPSAMPLE | 2× nearest-neighbor: each source pixel emitted 2×2 times into SRAM. Handles FPN neck upsample at layers 10 and 13. |
| Concat | CONCAT | 2D-strided gather: reads from two DDR4 base addresses and interleaves channels into one SRAM block. Used for all five FPN/PAN concat operations. |
| Coeff load | COEFF_LOAD | Write per-channel (M, S) requant coefficients from DDR4 into the Requant BRAM before each layer. |

# 3. Instruction Set Architecture (ISA)

## 3.1 Design Principles

- Fixed-width 128-bit instruction words — simplifies fetch alignment and FIFO sizing

- Opcode in the high byte [127:120] — sequencer decodes without full payload parse

- Unit ID in bits [119:116] — explicit routing to five per-unit command FIFOs

- Dependency flags always in bits [115:112] — uniform across all instruction types

- Remaining 112 bits are instruction-specific payload

## 3.2 Instruction Word Encoding

| Field | Bit Range | Description |
| --- | --- | --- |
| opcode | [127:120] | 8-bit opcode identifying the instruction type |
| unit_id | [119:116] | Target unit: 0=Sequencer, 1=DMA, 2=SA, 3=PSB/Requant, 4=VPU |
| dep_push_next | [115] | Push done-token to downstream unit’s RAW FIFO on completion |
| dep_push_prev | [114] | Push done-token to upstream unit’s WAR FIFO on completion |
| dep_pop_next | [113] | Block until token arrives from upstream unit’s RAW FIFO |
| dep_pop_prev | [112] | Block until token arrives from downstream unit’s WAR FIFO |
| payload | [111:0] | 112-bit instruction-specific data |

## 3.3 Complete Instruction Table

| Instruction | Opcode | Unit | Description |
| --- | --- | --- | --- |
| CONFIG | 0x01 | Sequencer | Load layer parameters into shadow registers: tile shape (M,N,K), conv stride, requant scale M/S base address, padding mode, activation type, pool size. |
| FENCE | 0x02 | Sequencer | Block until all units in a bitmask (bits [7:0] of payload) report completion. Used to synchronise pipeline stages at layer boundaries. Bitmask allows waiting on any combination of units simultaneously. |
| DMA_LOAD | 0x11 | DMA | 2D-strided fetch of INT8 activation tile from DDR4 into Act Bank. Descriptor: base_addr, row_stride, tile_w, tile_h, ch_count, pad counts. Zero-padding inserted in HW. |
| DMA_STORE | 0x12 | DMA | Burst-write INT8 Output Bank to DDR4. Used after VPU completes post-processing. |
| WT_LOAD | 0x10 | DMA | Prefetch INT8 weight tile from DDR4 into inactive Weight Bank (A or B). Runs concurrently with SA compute on active Weight Bank. |
| UPSAMPLE | 0x13 | DMA | 2× nearest-neighbor upsample. Reads source tensor from DDR4, emits each pixel 2×2 times into SRAM. Handles FPN neck upsample at layers 10 and 13. |
| CONCAT | 0x14 | DMA | 2D-strided gather: reads from two DDR4 base addresses (src_a, src_b) and interleaves channels into contiguous SRAM block. Used for all five FPN/PAN concat operations. |
| COEFF_LOAD | 0x15 | DMA | Write per-channel (M: INT16, S: UINT4) requant coefficient pairs from DDR4 into the Requant pipeline BRAM. Args: src DDR4 addr, channel count. |
| MATMUL | 0x20 | SA | Execute one 16×16 INT8 matrix multiply tile. Reads from Act Bank and Weight Bank; outputs INT32 results. PSB_ACC must immediately follow to accumulate. |
| PSB_ACC | 0x21 | PSB | Add current SA output into PSB INT32 running total. Issues after every MATMUL. No SRAM access. |
| PSB_FLUSH | 0x22 | PSB | Signal K-tiles complete. Forward final INT32 values from PSB to Requant pipeline input. Zero-clear PSB for next layer. |
| REQUANT | 0x30 | Requant | Apply per-channel (M, S) multiply-shift-clip to PSB output: clip(round((INT32×M)>>S), −128, 127). Reads (M,S) from coefficient BRAM sequentially per channel. |
| LUT_LOAD | 0x31 | VPU | Write 256 bytes of LUT data (one per INT8 input index) into Act LUT BRAM. Args: src DDR4 address. Must issue before SIMD_ACT on a new layer. |
| LUT_BYPASS | 0x32 | VPU | Enable or disable the LUT bypass mux. When enabled, Act LUT output is replaced by passthrough (for linear layers with no activation function). |
| SIMD_ACT | 0x33 | VPU | Run Act LUT lookup across all 64 lanes for one output row. INT8 input → LUT → INT8 output. Single-cycle throughput per lane after LUT_LOAD. |
| RELU | 0x34 | VPU | Apply ReLU: clamp INT8 values at zero. Simple per-lane comparator. No DSP/LUT required. For future YOLOv8 variants. |
| ELEW_ADD | 0x35 | VPU | Elementwise saturating INT8 add. Reads activated output and Residual Bank. Optional 1-bit arithmetic right shift to prevent overflow (controlled by CONFIG). For C2f residual connections. |
| ELEW_MUL | 0x36 | VPU | Elementwise INT8 multiply. INT8×INT8→INT16 intermediate, requantized to INT8 using layer M/S. Rarely used in YOLOv8n; included for future model support. |
| MAXPOOL | 0x37 | VPU | Sliding-window maximum per lane. 3×3 or 5×5 kernel (set via CONFIG pool_size bit). SPPF three-stage 5×5 pooling at layer 9. Tournament comparator tree: 25 inputs, 5 levels. |
| HREDUCE | 0x38 | VPU | 4-stage binary reduction tree within 16-element lane groups. Fires once per frame for DFL box decoding: max → subtract → exp LUT → sum → dot·[0..15] → continuous box coordinates. Passthrough (no-op) on all conv layers. |

## 3.4 Sample Microcode — One Conv+SiLU Layer

Full instruction sequence for a 3×3 conv, 128 input channels, 64 output channels, 40×40 spatial tile. BN pre-folded. 2D strided DMA fetches im2col layout natively.

COEFF_LOAD  src=DDR4:0x0034_0000  ch=64               ; per-channel (M, S)

LUT_LOAD    src=DDR4:0x0035_0000                       ; SiLU LUT for this layer

WT_LOAD     src=DDR4:0x0400_0000  dst=Weight_BankB     ; prefetch K=0..15 weights

DMA_LOAD    base=DDR4:0x0200_0000  row_stride=...      ; activation tile K=0

FENCE       wait=[DMA_done, WT_done]

MATMUL      act=ActBank_A  wt=WeightBank_B             ; K tile 0

PSB_ACC                                                 ; accumulate into PSB

WT_LOAD     src=DDR4:0x0400_0100  dst=Weight_BankA     ; prefetch K=16..31

DMA_LOAD    base=DDR4:0x0200_0100  row_stride=...      ; activation tile K=1

... (repeat for K=32..127, 8 tiles total)

MATMUL      act=ActBank_A  wt=WeightBank_B             ; final K tile

PSB_ACC

PSB_FLUSH                                               ; forward INT32 to Requant

FENCE       wait=[PSB_done]

REQUANT                                                  ; per-channel M/S clip

SIMD_ACT                                                 ; SiLU LUT lookup

DMA_STORE   src=OutputBank  dst=DDR4:0x0300_0000

# 4. YOLOv8n Layer Partition

YOLOv8n has 23 layers (indexed 0–22). Layers 0–21 run entirely on the FPGA NPU. Layer 22 (Detect head) runs on the ARM PS. ~95% of inference FLOPs are on the FPGA. BN eliminated via model.fuse().

## 4.1 Backbone (Layers 0–9) — All FPGA

| # | Operation | Input Shape | Output Shape | Unit |
| --- | --- | --- | --- | --- |
| 0 | Conv 3×3 s2 — 3→16 ch | 640×640×3 | 320×320×16 | SA + Requant + VPU |
| 1 | Conv 3×3 s2 — 16→32 ch | 320×320×16 | 160×160×32 | SA + Requant + VPU |
| 2 | C2f n=1 — 32→32 ch | 160×160×32 | 160×160×32 | SA + Requant + VPU + ELEW_ADD |
| 3 | Conv 3×3 s2 — 32→64 ch | 160×160×32 | 80×80×64 | SA + Requant + VPU |
| 4 | C2f n=2 — 64→64 ch | 80×80×64 | 80×80×64 | SA + Requant + VPU + ELEW_ADD |
| 5 | Conv 3×3 s2 — 64→128 ch | 80×80×64 | 40×40×128 | SA + Requant + VPU |
| 6 | C2f n=2 — 128→128 ch (P4 saved) | 40×40×128 | 40×40×128 | SA + Requant + VPU + ELEW_ADD |
| 7 | Conv 3×3 s2 — 128→256 ch | 40×40×128 | 20×20×256 | SA + Requant + VPU |
| 8 | C2f n=1 — 256→256 ch | 20×20×256 | 20×20×256 | SA + Requant + VPU + ELEW_ADD |
| 9 | SPPF — 3× MaxPool 5×5, concat ×4 ch | 20×20×256 | 20×20×256 | VPU (MAXPOOL ×3) |

## 4.2 FPN/PAN Neck (Layers 10–21) — All FPGA

| # | Operation | Input Shape | Output Shape | Unit |
| --- | --- | --- | --- | --- |
| 10 | Upsample 2× nearest-neighbor | 20×20×256 | 40×40×256 | DMA (UPSAMPLE) |
| 11 | Concat L10 + L6 skip | 40×40×256 + 40×40×128 | 40×40×384 | DMA (CONCAT) |
| 12 | C2f n=1 — 384→128 ch | 40×40×384 | 40×40×128 | SA + Requant + VPU |
| 13 | Upsample 2× nearest-neighbor | 40×40×128 | 80×80×128 | DMA (UPSAMPLE) |
| 14 | Concat L13 + L4 skip | 80×80×128 + 80×80×64 | 80×80×192 | DMA (CONCAT) |
| 15 | C2f n=1 — 192→64 ch (P3 out) | 80×80×192 | 80×80×64 | SA + Requant + VPU |
| 16 | Conv 3×3 s2 — 64→64 ch | 80×80×64 | 40×40×64 | SA + Requant + VPU |
| 17 | Concat L16 + L12 skip | 40×40×64 + 40×40×128 | 40×40×192 | DMA (CONCAT) |
| 18 | C2f n=1 — 192→128 ch (P4 out) | 40×40×192 | 40×40×128 | SA + Requant + VPU |
| 19 | Conv 3×3 s2 — 128→128 ch | 40×40×128 | 20×20×128 | SA + Requant + VPU |
| 20 | Concat L19 + L9 skip | 20×20×128 + 20×20×256 | 20×20×384 | DMA (CONCAT) |
| 21 | C2f n=1 — 384→256 ch (P5 out) | 20×20×384 | 20×20×256 | SA + Requant + VPU |

## 4.3 Detect Head (Layer 22) — ARM + HREDUCE assist

| # | Operation | Inputs | Notes |
| --- | --- | --- | --- |
| 22 | Detect [L15, L18, L21] — DFL + NMS | 80×80×64, 40×40×128, 20×20×256 | HREDUCE in VPU performs 4-stage tree reduction for DFL box decode. Class sigmoid and NMS run on A53 in PyTorch. Total: ~3–8 ms. |

## 4.4 Skip-Connection Feature Map Retention

| Layer | Tensor | Size (INT8) | Retained Until |
| --- | --- | --- | --- |
| L4 output | 80×80×64 | 409 KB | Layer 14 concat |
| L6 output | 40×40×128 | 205 KB | Layer 11 concat |
| L9 output | 20×20×256 | 103 KB | Layer 20 concat |

Total live DDR4 retention: ~717 KB. These regions are reserved in the DDR4 memory map and never overwritten by subsequent DMA_STORE operations.

# 5. Memory Architecture

## 5.1 Memory Hierarchy

- DDR4 (4 GB) — all weights, all activations, NPU instruction programs, requant coefficient table

- AXI HP ports — 64-bit read/write channels at up to ~10 GB/s sustained per port

- On-chip SRAM (~38 BRAM36 + URAM candidate for Residual Bank) — Act A/B, Weight A/B, Residual, Output

- Partial Sum Buffer (PSB, ~1 KB BRAM/registers) — INT32 accumulation across K tiles

- Requant BRAM (~1 BRAM18) — per-channel (M, S) coefficient store

## 5.2 DDR4 Memory Map

| Region | Address Range | Size / Notes |
| --- | --- | --- |
| NPU instruction programs | 0x2000_0000 — 0x2000_FFFF | 64 KB — per-inference instruction stream, written by driver |
| INT8 model weights | 0x2001_0000 — 0x2033_FFFF | ~3.2 MB — all 22 FPGA layers, packed INT8 |
| Requant coefficient table | 0x2034_0000 — 0x2034_FFFF | 64 KB — per-layer per-channel (M: INT16, S: UINT4) pairs |
| Input image buffer | 0x2035_0000 — 0x2035_FFFF | 640×640×1 = 409 KB; uint8 raw, INT8 after ARM preprocessing |
| Layer activation buffers (ping-pong) | 0x2040_0000 — 0x2060_FFFF | ~33 MB — double-buffered output activations, no im2col workspace |
| Skip feature maps (L4, L6, L9) | 0x2061_0000 — 0x2062_FFFF | ~717 KB — reserved, never overwritten mid-inference |
| SiLU LUT store (per layer) | 0x2063_0000 — 0x2063_FFFF | 64 KB — one 256-entry LUT per layer pre-computed offline |
| ARM output buffer (detect head) | 0x2064_0000 — 0x2064_FFFF | 64 KB — bbox + class outputs from NMS |
| Unused / reserved | 0x2065_0000 — end | Remainder of 4 GB for OS, kernel, etc. |

Note: The 40 MB im2col workspace present in v1.0 is eliminated. The 2D strided DMA fetches patches directly from the activation buffers with no intermediate DDR4 write.

# 6. Pipelining and Concurrency

## 6.1 Three-Stage Pipeline

| Time Slot | DMA | SA + PSB | Requant + VPU | ARM PS |
| --- | --- | --- | --- | --- |
| T=0 | WT_LOAD + DMA_LOAD L0 | filling banks | idle | idle |
| T=1 | WT_LOAD + DMA_LOAD L1 | MATMUL L0 × K | idle | idle |
| T=2 | WT_LOAD + DMA_LOAD L2 | MATMUL L1 × K | REQUANT + SiLU L0 | idle |
| T=3 | DMA_STORE L0 | MATMUL L2 × K | REQUANT + SiLU L1 | idle |
| T=N | DMA_STORE L21 | done | done | HREDUCE + NMS detect |

## 6.2 FENCE Usage

- FENCE(DMA_done + WT_done) before MATMUL: waits for activation and weight loads before SA reads

- FENCE(PSB_done) before REQUANT: waits for PSB_FLUSH before Requant reads INT32 values

- FENCE(VPU_done) before DMA_STORE: waits for SiLU/eltwise before DMA reads Output Bank

- No FENCE between WT_LOAD L(n+1) and MATMUL L(n) — they target different Weight Banks

## 6.3 Bottleneck Analysis at 300 MHz

For a 40×40×128 layer:

- DMA load (2D strided, no im2col): ~0.3 ms AXI burst only — no ARM overhead

- Systolic compute: (40×40×128 / 256 MACs) / 300 MHz ≈ 1.1 ms

- Requant + VPU: 40×40×64 elements / 64-lane / 300 MHz ≈ 0.4 ms

The SA remains the bottleneck. DMA and VPU together are comfortably faster, so pipeline efficiency is high. The v1.0 ARM im2col overhead (~5 ms/frame) is fully eliminated.

# 7. KR260 Xilinx IP Cores

| IP Block | Xilinx IP Core | Purpose |
| --- | --- | --- |
| Zynq UltraScale+ MPSoC | Processing System (PG) block design PS cell | ARM A53, DDR4 controller, AXI HP0/HP1, AXI-Lite GP, GIC |
| AXI SmartConnect | PG247 | Routes PS GP → NPU CSR; routes HP0/HP1 → DMA engine |
| Block Memory Generator | PG058 | Act Banks A/B, Weight Banks A/B, Residual Bank, Output Bank, Requant coeff BRAM, SiLU LUT (multiple instances) |
| FIFO Generator | PG057 | Dependency token FIFOs (depth=4, 1-bit); skew FIFOs for SA column alignment (depth=0..15, 8-bit) |
| Integrated Logic Analyzer | PG172 (ILA) | Debug probes on AXI HP bus, SRAM addr/data, sequencer FSM, PSB flush, dep-FIFO tokens |
| Virtual I/O | PG159 (VIO) | Runtime injection: force fence, reset individual units, single-step sequencer |
| AXI GPIO | PG144 | Expose sequencer IRQ and global NPU reset to PS |
| Clocking Wizard | PG065 | Generate 300 MHz NPU PL clock; 100 MHz AXI-Lite CSR clock |
| DSP48E2 | Primitive (no PG) | Direct instantiation in systolic PE Verilog for precise INT8 packing control |

## 7.1 AXI Port Configuration

- S_AXI_HP0_FPD: 128-bit, connected to DMA engine read port (weight + activation loads from DDR4)

- S_AXI_HP1_FPD: 128-bit, connected to DMA engine write port (activation store to DDR4)

- S_AXI_GP0_FPD (M_AXI_GP0): 32-bit, connected to NPU AXI-Lite CSR block

- IRQ_F2P[0]: connected to sequencer program-complete interrupt output

Keep HP0 and HP1 on separate ports to allow simultaneous read+write. Sharing a single HP port causes head-of-line blocking that reduces effective bandwidth by up to 40%.

# 8. Software Stack

## 8.1 Driver Interface (AXI-Lite CSR Map)

v2.1: AXI-Lite slave is write-only (AW + W + B channels only; AR/R omitted). The ARM driver never reads CSR registers back -- it writes INSTR_BASE, INSTR_COUNT, and INSTR_KICK once per layer dispatch, then waits on the completion IRQ. NPU_VERSION, NPU_STATUS, and IRQ_STATUS from v2.0 are removed.

| Register | Offset | Description |
| --- | --- | --- |
| NPU_CTRL | 0x008 | Write-only. Bit 0: global reset (self-clearing). Bit 1: sequencer kick -- 1-cycle strobe that latches INSTR_BASE and INSTR_COUNT and begins fetch. No level register; ARM does not need to clear it. |
| INSTR_BASE_LO | 0x010 | Low 32 bits of DDR4 physical address of instruction program buffer |
| INSTR_BASE_HI | 0x014 | High 32 bits of DDR4 physical address |
| INSTR_COUNT | 0x018 | Number of 128-bit instructions in program buffer |

## 8.2 Linux Driver Outline

The kernel driver (/dev/mynpu) is a platform character device providing:

- mmap the AXI-Lite CSR region and DDR4 NPU workspace into kernel virtual address space on probe

- ioctl interface: NPU_IOC_RUN (pass instruction buffer, wait for IRQ), NPU_IOC_STATUS

- dma_alloc_coherent for instruction buffer and NPU DDR4 workspace

- IRQ handler for program-complete interrupt using wait_for_completion / complete

- Single completion interrupt per frame — all sub-unit interrupts masked in driver

## 8.3 PyTorch Integration

import torch

torch.ops.load_library('libnpu_ops.so')

torch.library.define('mynpu::conv_int8',

'(Tensor x, Tensor w, int stride, float scale_in,

Tensor M, Tensor S, float scale_out) -> Tensor')

The op: opens /dev/mynpu, writes COEFF_LOAD + LUT_LOAD + instruction stream via ioctl, blocks on IRQ, returns output tensor. torch.compile replaces Conv+SiLU, ELEW_ADD, UPSAMPLE, CONCAT subgraphs automatically.

# 9. Quantization and Supporting Details

## 9.1 Post-Training Quantization (PTQ)

Run 500–1000 calibration images through the FP32 model. For each Conv layer record the per-channel min/max of output activations. Compute per-channel scale: scale_c = max(|min_c|, |max_c|) / 127.0. Symmetric INT8, zero_point = 0. Derive (M, S) per output channel using Jacob et al. fixed-point formulation from the product of input scale, weight scale, and output scale.

BN Folding (required before deployment): call model.fuse() on the YOLOv8 model before quantization. After fusing, every Conv module has bias=True and no BatchNorm child. The folded bias is absorbed into the (M, S) pair in the Requant pipeline.

## 9.2 SiLU LUT Generation

def gen_silu_lut(scale_in, scale_out):

x_fp = np.arange(-128, 128, dtype=np.float32) * scale_in

y_fp = x_fp * torch.sigmoid(torch.tensor(x_fp)).numpy()

y_int = np.clip(np.round(y_fp / scale_out), -128, 127).astype(np.int8)

return y_int  # 256-entry LUT, 1 byte each

One LUT per layer (same across all 64 lanes). Written to DDR4 SiLU LUT store, then loaded to VPU BRAM via LUT_LOAD instruction at layer start.

## 9.3 Input Preprocessing

ARM NEON-vectorised conversion before writing to input buffer: int8_val = round(uint8_val × (scale_int8 / 255.0) − 128). Single pass over 640×640×3 = 1.2 MB, ~0.15 ms on A53 with NEON.

## 9.4 Interrupt Coalescing

Sequencer asserts program-complete IRQ only when the final DMA_STORE instruction completes. All sub-unit interrupts are masked in the driver. ARM waits on a single wait_for_completion() per inference frame. Overhead: ~5–10 us per frame (one context switch) vs ~50–100 us for 22 per-layer interrupts in v1.0.

## 9.5 Future: INT8 DSP Dual-Packing (v2 SA)

Once v2.0 end-to-end correctness is validated, the systolic array can adopt Xilinx WP486 dual-packing: two INT8 multiplications per DSP48E2 per cycle using the pre-adder path. This halves DSP count from 256 to 128 while maintaining throughput (~204 GOPS at 300 MHz), or doubles throughput to ~204 GOPS.

## 9.6 Open Questions

| Open Question | Status / Notes |
| --- | --- |
| Residual Bank sizing | 80×80×64 skip tensor = 409 KB. Exceeds typical BRAM budget. Consider URAM or DDR4 round-trip with DMA prefetch. Design decision pending. |
| SPPF concat implementation | 4-way concat along channel dim — needs CONCAT instruction three times or a dedicated 4-way DMA mode. Design decision pending. |
| Tiling for 80×80 layers | 80×80×192 activations may not fit in Act Banks in one shot. Tiling along H/W introduces halo overlap. Halo handling strategy TBD. |
| QAT vs PTQ final decision | Run PTQ first; if mAP drop > 1.5 points, escalate to QAT (~50 GPU-hours on A100). |
| Timing closure at 300 MHz | XCK26 achievable for well-pipelined DSP48E2 chains. Verify after RTL implementation. 250 MHz fallback if closure fails. |
| HREDUCE scheduling | HREDUCE fires once per frame on final layer. Verify it does not stall VPU pipeline on layer 21 output. Explicit FENCE before HREDUCE required. |

# 10. Implementation TODO List

## 10.1 Quantization & Model Export

- Export YOLOv8n from PyTorch using torch.export or ONNX

- Run PTQ with 500+ representative COCO images

- Extract per-channel (M, S) scale/shift pairs for every Conv2d layer

- Fuse BatchNorm into Conv2d weights and bias offline (model.fuse())

- Build per-layer per-channel quantization config file (JSON/YAML)

- Precompute SiLU LUT entries and quantize to INT8

- Validate INT8 model accuracy vs FP32 baseline (target: <1.0 mAP drop)

## 10.2 RTL — 2D Strided DMA

- Design DMA descriptor register file (base_addr, row_stride, tile_w, tile_h, ch_count, pad mask)

- Implement row-stride address generator with automatic gap-skip

- Implement zero-padding insertion state machine (suppress DRAM read, output zero word)

- Implement AXI4 master read/write channels (HP0 read, HP1 write)

- Implement UPSAMPLE and CONCAT modes

- Implement ping-pong bank select logic for Act and Weight Banks

- Write testbench: verify tile extraction at image edges with padding

## 10.3 RTL — SRAM Hub

- Instantiate Act Banks A/B (dual-port BRAM, read: SA, write: DMA)

- Instantiate Weight Banks A/B (dual-port BRAM, read: SA load phase, write: DMA WT_LOAD)

- Instantiate Residual Bank (write: DMA, read: VPU eltwise — independent port from Act Banks)

- Instantiate Output Bank (write: VPU, read: DMA_STORE)

- Implement bank-select mux and ping-pong swap control signals

## 10.4 RTL — Systolic Array 16×16

- Implement single INT8 PE: registered multiply-accumulate into INT32

- Tile 16×16 PE array with weight-stationary datapath

- Implement activation skew-load FIFOs (depth = column index)

- Implement weight load phase from Weight Bank to PEs

- Wire SA output (256 INT32 values) to PSB_ACC input

- Confirm DSP48E2 dual-packing for v2 (two MACs per DSP)

- Write testbench: verify INT32 output against numpy reference for 3×3 and 1×1 conv tiles

## 10.5 RTL — PSB

- Implement 16×16 INT32 register bank (one accumulator per PE output)

- Implement PSB_ACC: add new SA output to running total

- Implement PSB_FLUSH: forward final INT32 to Requant; zero-clear for next layer

- Write testbench: multi-tile accumulation matches reference

## 10.6 RTL — Requant Pipeline

- Implement per-channel coefficient BRAM (512 × {M: INT16, S: UINT4})

- Implement COEFF_LOAD write path from DMA into BRAM

- Implement sequential read: increment channel address each output cycle

- Implement multiply-shift pipeline: INT32 × INT16 → INT48, right-shift by S, round

- Implement saturating clip to [−128, 127]

- Write testbench: verify per-channel output matches software PTQ reference

## 10.7 RTL — VPU (64 lanes)

- Implement 64-lane datapath with Act LUT BRAM and LUT_BYPASS mux

- Implement SIMD_ACT: LUT lookup per lane

- Implement ELEW_ADD: saturating lane-wise add from Residual Bank

- Implement ELEW_MUL: INT8×INT8 → INT16 → requantized INT8

- Implement RELU: max(0, x) per lane

- Implement MAXPOOL: sliding 5×5 window, tournament comparator tree

- Implement HREDUCE: 4-stage binary reduction tree (16→08→04→02→1), exp LUT, dot·[0..15]

- Write testbench: SiLU correctness, ELEW_ADD saturation, HREDUCE vs numpy softmax

## 10.8 RTL — Sequencer

- Implement five instruction FIFOs: DMA, SA, PSB, Requant, VPU

- Implement 128-bit instruction decoder: opcode + unit_id + dep flags + payload

- Implement FENCE barrier: stall fetch until unit_done bitmask clears

- Implement global stall: when current target FIFO is full, hold fetch FSM; do not advance instr_ptr

- Implement AXI-Lite write-only slave (AW+W+B channels; no read channels): decode INSTR_BASE_LO, INSTR_BASE_HI, INSTR_COUNT, INSTR_KICK

- Implement completion IRQ: fire on final DMA_STORE done

- Write testbench: full single-layer dispatch sequence

## 10.9 Integration & Top-Level

- Wire all blocks at top level

- Connect AXI-Lite slave to Zynq PS AXI master in Vivado block design

- Connect AXI4 DMA read master to HP0; write master to HP1

- Assign AXI-Lite CSR base address in device tree

- Run Vivado synthesis and check timing at 300 MHz

- Fix timing violations; verify resource utilisation fits KR260

## 10.10 Verification

- Unit testbench per RTL block (cocotb + Verilator preferred)

- Full-layer cocotb testbench: feed real INT8 YOLOv8n Conv2d tile, compare to PyTorch INT8 reference

- Verify 2D strided DMA padding correctness on all four image edges

- Verify K-tile accumulation across multiple PSB_ACC / PSB_FLUSH cycles

- Verify per-channel Requant matches PTQ reference values

- Verify SiLU LUT matches software reference across all 256 INT8 inputs

- Verify HREDUCE softmax output matches numpy softmax

- Run ILA capture on hardware for a single layer dispatch

- Acceptance criterion: mAP50 on COCO val2017 (500 images) within 1.5 points of FP32 baseline

## 10.11 Linux Driver

- Write Linux char driver (/dev/npu0) as platform character device

- Map AXI-Lite CSR region with ioremap

- Allocate DMA-coherent buffers with dma_alloc_coherent

- Implement ioctl: LOAD_WEIGHTS, DISPATCH_LAYER, WAIT_DONE

- Register IRQ handler for completion interrupt

- Write device tree node (reg, interrupts, compatible = ‘mynpu,v2’)

## 10.12 PyTorch Integration

- Register torch.library custom op: npu::conv_silu(input, weight, M, S, lut) -> Tensor

- Implement op: copy tensors → write COEFF_LOAD + LUT_LOAD + layer descriptor → wait on IRQ → return output

- Register autograd passthrough (inference only)

- Write Python layer dispatch scheduler: iterate YOLOv8n graph, call NPU op per Conv2d

- Benchmark end-to-end latency: preprocess → NPU → HREDUCE → NMS

## 10.13 Bring-Up Sequence

- Flash bitstream to KR260 via Vivado Hardware Manager over Tailscale / JTAG

- Confirm AXI-Lite register read/write from ARM (devmem2 smoke test)

- Run single-layer dispatch: verify Output Bank contents via ILA

- Run full YOLOv8n inference on a single test image

- Compare NPU output vs PyTorch CPU INT8 reference (check mAP)

- Measure per-frame latency and throughput at 300 MHz

- Profile bottlenecks: DMA bandwidth, SA utilisation, VPU idle time

# 11. Key References

| Reference | Relevance |
| --- | --- |
| Jacob et al., Quantization and Training of Neural Networks for Efficient Integer-Arithmetic-Only Inference, CVPR 2018 (arXiv:1712.05877) | INT8 per-channel requantization M × 2^(−n) scheme; BN folding math |
| Xilinx WP486: Inference of Neural Networks Using INT8 on Xilinx UltraScale+ MPSoC | INT8 dual-packing for DSP48E2; latency benchmarks on ZU5EV |
| Gemmini (UC Berkeley): github.com/ucb-bar/gemmini | Weight-stationary systolic array RTL reference; scratchpad memory architecture; 2D-strided DMA patterns |
| Apache TVM VTA: github.com/apache/tvm-vta | Microcoded sequencer; dependency FIFO design; instruction encoding reference |
| NVDLA: github.com/nvdla/hw | AXI4 master DMA patterns; SDP post-processing pipeline; CSR register map conventions |
| Eyeriss v2 (MIT): arXiv:1807.07928 | Flexible dataflow; depthwise conv handling for future variants |
| Ultralytics YOLOv8 YAML: github.com/ultralytics/ultralytics | Authoritative layer definitions; depth_multiple and width_multiple for ‘n’ variant |
| Xilinx PG058, PG057, PG247, PG172 | BRAM Generator, FIFO Generator, SmartConnect, ILA product guides |
| Xilinx UG1085: Zynq UltraScale+ Device TRM | AXI HP port specs; DDR4 controller latency; PS-PL interface constraints |
| Xilinx dma-proxy driver | Linux DMA coherency patterns; dma_alloc_coherent usage for AXI HP transfers |

# 12. Changes from v2.0

## 12.1 Sequencer Stall Policy — Global Stall on Target FIFO Full

Decision: When the sequencer’s current instruction targets a unit FIFO that is full, the sequencer halts DDR4 fetch entirely and retries every cycle until the push succeeds. It does not skip ahead or reorder instructions.

Stall condition (RTL):

assign dispatch_stall = (current_unit != UNIT_SEQ) & fifo_full[current_unit_idx];

FENCE and CONFIG (unit_id == UNIT_SEQ) are never pushed to a unit FIFO and are exempt from fifo_full stalls.

Why not per-unit lookahead / out-of-order dispatch: An out-of-order dispatcher would need to buffer fetched-but-not-yet-dispatched instructions in a secondary FIFO, maintain a per-unit availability scoreboard, and verify that skipping instruction N does not violate the ordering assumptions the ARM driver used when encoding the dep token bits. The ARM driver was written assuming strict in-order issue — no instruction in the stream was encoded with dep tokens conservative enough to tolerate reordering. Building a reorder buffer to safely skip ahead would add ~100–200 LUTs of scoreboard logic, a prefetch FIFO, and a hazard detection unit, for a correctness risk that isn’t worth the marginal throughput gain. [Source: apache/tvm-vta — VTA fetch FSM stalls on cmd_queue_full, never reorders.]

Why global stall is safe and sufficient: The per-unit FIFOs are sized to absorb the sequencer-to-unit speed mismatch (DMA=16, SA=32, PSB=32, REQ=8, VPU=16). At 300 MHz, with FENCE barriers resetting FIFO fill at every layer boundary, a FIFO-full stall is uncommon in the YOLOv8n instruction stream. When it does occur, it stalls only for the cycles needed to drain one slot — the unit FSMs are already pulling concurrently. The dep token system continues to provide cross-unit parallelism regardless of whether the sequencer is momentarily stalled. [Source: ISA_Architecture_QA.md §11, §5.]

RTL resource cost: One 128-bit shift register (4 × 32-bit FFs) for in-flight instruction assembly. No prefetch FIFO. No scoreboard.

## 12.2 AXI-Lite Slave — Write-Only, Read Channels Removed

Decision: The AXI-Lite slave on the sequencer implements only the AW + W + B channels (write-only). The AR and R channels are omitted entirely. s_axil_wstrb is also omitted. The CSR map is simplified to three write-only registers: INSTR_BASE (split across INSTR_BASE_LO and INSTR_BASE_HI for the 64-bit DDR4 pointer), INSTR_COUNT, and INSTR_KICK. NPU_VERSION, NPU_STATUS, and IRQ_STATUS from v2.0 are removed.

Reasoning: The CSR registers are write-only from the ARM’s perspective. The driver writes them once per layer dispatch and waits on the completion IRQ — it never reads them back. Implementing the AR and R channels adds 7 signals to the port list and requires read-address handshake logic and a read-data mux for zero benefit. INSTR_KICK is implemented as a write-triggered 1-cycle internal strobe rather than a level signal, eliminating the need for the ARM to explicitly clear it with a second transaction (which would be required with AXI GPIO — ruled out for this reason). [Source: ISA_Architecture_QA.md §22; ZipCPU easyaxil — write-only slave pattern.]

— End of Document —